# LAB 6: System Analysis & Planning

Subject: System Simulation Lab / Capstone Project

Submission Date: 23 August 2026

Project Title: Intelligent Trip Planning for Battery Electric Vehicles Using Real-Time Map Data

Challenge Reference: MathWorks Excellence in Innovation Challenge — Project #255

Institution: Vidyalankar Institute of Technology (VIT), Mumbai

Department: Department of Electronics and Computer Science Engineering

### Team Members & Contributors:

1. Rajiv Gohil (Project Lead, Powertrain Dynamics & Simulink, Route Engine) — rajiv.gohil24@vit.edu.in

2. Sneha Pardeshi (Google Maps API & Driving Cycles, Energy Accounting, Charging Optimization & UI) — sneha.pardeshi@vit.edu.in

## 1. Project Overview & Motivation

As electric vehicles (EVs) become central to sustainable transportation, accurate trip planning and energy management are paramount. Standard homologation cycles (such as NEDC or WLTP) fail to capture real-world driving conditions, leading to unexpected range depletion and "range anxiety" for EV drivers. Real-world energy consumption is heavily dictated by:

Topographical elevation variations (hill climbs and descents),

Dynamic traffic bottlenecks and speed fluctuations,

Speed limits and aerodynamic drag scaling quadratically with speed,

Ambient temperature variations and auxiliary HVAC electrical loads.

This project designs a simulation-based workflow for Battery Electric Vehicles (BEVs) that couples real-time mapping data (via Google Maps API) with detailed longitudinal powertrain physics and equivalent circuit battery models in MATLAB® and Simulink®.

## 2. System Requirements Specification

### 2.1 Functional Requirements (FR)

FR-1 (Real-Time Route Ingestion): Interface MATLAB with Google Maps Directions & Elevation APIs to extract waypoint coordinates, route distance, step duration, legal speed limits, and continuous elevation profiles for any origin-destination pair.

FR-2 (Offline Fallback Capability): Provide high-resolution pre-compiled datasets for key Indian highway corridors (e.g., Mumbai-Pune Expressway, Old NH48, Yamuna Expressway) ensuring zero-dependency operation.

FR-3 (Driving Cycle Synthesizer): Synthesize a continuous 2 Hz time-speed trajectory $[t, v(t), \theta(t)]$ reflecting traffic flow and road grade, formatted as a standard MATLAB timeseries and MAT-file for the Simulink Driving Cycle Source block.

FR-4 (BEV Modeling via Virtual Vehicle Composer): Define customizable BEV models (curb mass, aerodynamic drag $C_d$, frontal area $A$, rolling resistance $C_{rr}$, motor envelope, and battery capacity) parameterized matching MathWorks Virtual Vehicle Composer.

FR-5 (Powertrain Simulation & Energy Accounting): Accurately compute tractive forces, motor electrical losses, regenerative braking recovery, net energy consumption (kWh), efficiency (kWh/100 km), battery State of Charge (SOC) trajectory, and operational electricity costs in INR (₹).

FR-6 (Driving Modes Optimization): Implement user-selectable driving modes (Eco, Normal, Performance) with dynamic acceleration constraints, cruising speed caps, regen prioritization, and HVAC power scaling.

FR-7 (Charging Stop Optimization): Detect battery reserve boundary violations ($SOC < 15\%$), search for candidate DC fast charging stations along the route corridor, and compute CC-CV charging durations to 80% SOC.

FR-8 (Interactive Visualization & Reporting): Render interactive maps (satellite imagery and roadmap tiles) with elevation profiles and provide a one-click PDF summary report export.

### 2.2 Non-Functional Requirements (NFR)

NFR-1 (Engineering Fidelity): Energy estimations must align within $\pm 5\%$ of empirical on-road BEV test data.

NFR-2 (Execution Speed): Analytical route simulation solver must complete a 150 km corridor evaluation in under 3 seconds.

NFR-3 (Modularity & Maintainability): Decoupled architecture separating Route Engine, Vehicle Database, Driving Cycle Generator, Powertrain Physics, and UI.

## 3. Mathematical & Theoretical Modeling Framework

flowchart LR A["Road Coordinates & Elevation (Google Maps)"] --> B["Driving Cycle Synthesizer [t, v(t), grade]"] B --> C["Longitudinal Vehicle Dynamics (F_net = m*a)"] C --> D["Electric Motor Map (T, omega, Losses)"] D --> E["Battery ECM (Voc - I*Rint)"] E --> F["KPIs: SOC(t), Net kWh, Cost INR"]

### 3.1 Longitudinal Vehicle Dynamics

The dynamic force equilibrium along the vehicle's longitudinal axis is governed by Newton's Second Law:

$$m_{\text{eff}} \frac{dv}{dt} = F_{\text{traction}} - F_{\text{rolling}} - F_{\text{aero}} - F_{\text{grade}} - F_{\text{frictionBrake}}$$

Where:

1. Aerodynamic Drag:

$$F_{\text{aero}} = \frac{1}{2} \rho_{\text{air}} C_d A_{\text{frontal}} v^2$$

2. Tire Rolling Resistance:

$$F_{\text{rolling}} = C_{rr} m_{\text{veh}} g \cos(\theta) \tanh\left(\frac{v}{0.1}\right)$$

3. Road Grade Force:

$$F_{\text{grade}} = m_{\text{veh}} g \sin(\theta) \quad \text{where } \theta = \arctan\left(\frac{\text{grade}_{\%}}{100}\right)$$

4. Effective Rotational Inertia:

$$m_{\text{eff}} = m_{\text{veh}} + \frac{4 J_{\text{wheel}} + J_{\text{motor}} N_{\text{gear}}^2}{r_{\text{wheel}}^2}$$

### 3.2 Electric Motor & Inverter Loss Model

Motor torque is constrained by the base-speed field-weakening envelope:

$$T_{\text{max}}(\omega) = \begin{cases} T_{\text{peak}} & \omega \le \omega_{\text{base}} \\ \frac{P_{\text{peak}}}{\omega} & \omega > \omega_{\text{base}} \end{cases}$$

Motor losses are decomposed into copper heating, iron hysteresis/eddy currents, and windage friction:

$$P_{\text{loss}}(\omega, T) = k_{\text{cu}} T^2 + k_{\text{fe}} \omega + k_w \omega^3 + P_{\text{fixed}}$$

### 3.3 High-Voltage Battery Equivalent Circuit Model (ECM)

The traction pack is modeled with an open-circuit voltage $V_{\text{oc}}(\text{SOC})$ and internal resistance $R_{\text{int}}(\text{SOC}, T)$:

$$V_{\text{terminal}} = V_{\text{oc}}(\text{SOC}) - I_{\text{batt}} R_{\text{int}}(\text{SOC}, T_{\text{batt}})$$

$$I_{\text{batt}} = \frac{V_{\text{oc}} - \sqrt{V_{\text{oc}}^2 - 4 R_{\text{int}} P_{\text{batt}}}}{2 R_{\text{int}}}$$

$$\text{SOC}(t) = \text{SOC}_0 - \int_0^t \frac{I_{\text{batt}}(\tau)}{3600 \cdot Q_{\text{capacity}}} \, d\tau$$

## 4. Proposed System Architecture

flowchart TD subgraph Data Layer GDir["Google Directions API"] GElev["Google Elevation API"] OffData["Offline Corridors Database"] VehDB["Indian BEV Database (10 Models)"] end subgraph Processing Layer REng["Route Engine (Smoothing, Grade, Traffic)"] DCGen["Driving Cycle Synthesizer (Eco, Normal, Perf)"] end subgraph Simulation Layer VVC["Virtual Vehicle Composer Interface"] SimBEV["Simulink BEV Model (BEV.slx)"] AnalytSolver["Fast Analytical Powertrain Solver"] end subgraph Optimization & UI Layer OptEngine["Charging & Mode Optimizer"] MapDash["Interactive Satellite/Roadmap UI"] PDFGen["Automated PDF Report Generator"] end GDir --> REng GElev --> REng OffData -.-> REng REng --> DCGen DCGen --> SimBEV DCGen --> AnalytSolver VehDB --> VVC VVC --> SimBEV VehDB --> AnalytSolver AnalytSolver --> OptEngine OptEngine --> MapDash MapDash --> PDFGen

## 5. Work Breakdown Structure (WBS) & Task Allocation

| Module / Subsystem | Primary Owner | Secondary Contributor | Key Deliverables |
| --- | --- | --- | --- |
| Powertrain Modeling & Dynamics | Rajiv Gohil | Sneha Pardeshi | Longitudinal forces equation, rotational inertia, Simscape/Simulink subsystem setup (BEV.slx). |
| Route Engine & Google Maps API | Rajiv Gohil | Sneha Pardeshi | Google Directions/Elevation REST client, offline fallback corridor generator, spatial interpolation. |
| Driving Cycle Synthesis | Sneha Pardeshi | Rajiv Gohil | 2 Hz speed synthesizers, legal limit clamps, stop dwell models, Simulink timeseries exporter. |
| Energy Accounting & Costing | Sneha Pardeshi | Rajiv Gohil | Gross vs net kWh numerical integration, regen blending, HVAC auxiliary models, INR billing calculation. |
| Charging & Mode Optimization | Sneha Pardeshi | Rajiv Gohil | Reserve boundary monitor (SOC < 15\%), highway DC fast charger lookup, CC-CV taper duration algorithm. |
| Dashboard UI & Reporting | Sneha Pardeshi | Rajiv Gohil | App Designer UI, Google Maps satellite/roadmap tiles, Mapping Toolbox plotting, automated PDF generator. |
| Verification & Testing | Rajiv Gohil | Sneha Pardeshi | 6 automated unit test suites (runAllTests.m), baseline numerical benchmarking. |

## 6. Project Timeline & Milestone Schedule

gantt title BEV Intelligent Trip Planning Development Timeline dateFormat YYYY-MM-DD section Phase 1: Planning & Architecture Problem Analysis & Requirements (LAB 6) :done, 2026-08-10, 2026-08-23 section Phase 2: Core Prototype Development Google Maps API & Route Engine Setup :done, 2026-08-24, 2026-08-27 Driving Cycle Synthesizer & Exporter :done, 2026-08-25, 2026-08-28 Working Prototype & Initial UI (LAB 7) :done, 2026-08-26, 2026-08-29 section Phase 3: Advanced Optimization Simulink BEV Model & Virtual Vehicle Prep :done, 2026-08-30, 2026-09-02 Driving Modes & Charging Planner (LAB 8) :done, 2026-09-01, 2026-09-05 section Phase 4: Finalization & Evaluation Verification Testing & Slide Deck (LAB 9) :done, 2026-09-06, 2026-09-19 Final Report & Submission Proof (LAB 10) :done, 2026-09-20, 2026-09-26

## 7. Sign-off & Faculty Review

Student Signature (Lead): Rajiv Gohil

Student Signature (Member): Sneha Pardeshi

Faculty Guide / Reviewer: Dept. of Electronics and Computer Science Engineering, VIT Mumbai

Review Status: Accepted & Approved for Prototype Development