# LAB 8: Project Progress Report

Subject: System Simulation Lab / Capstone Project

Submission Date: 05 September 2026

Project Title: Intelligent Trip Planning for Battery Electric Vehicles Using Real-Time Map Data

Challenge Reference: MathWorks Excellence in Innovation Challenge — Project #255

Institution: Vidyalankar Institute of Technology (VIT), Mumbai

Department: Department of Electronics and Computer Science Engineering

### Team Members:

1. Rajiv Gohil (Project Lead, Powertrain Dynamics & Simulink, Route Engine) — rajiv.gohil24@vit.edu.in

2. Sneha Pardeshi (Google Maps API & Driving Cycles, Energy Accounting, Charging Optimization & UI) — sneha.pardeshi@vit.edu.in

## 1. Executive Summary & Project Status

Since the submission of the initial working prototype in LAB 7, the project has progressed into the Advanced Optimization & Validation Phase. All 5 Core Project Tasks are complete, and both optional Advanced Extensions have been implemented in the codebase.

### Progress Summary Dashboard

| Task / Module | Target Milestone | Status | % Completion |
| --- | --- | --- | --- |
| Core Task 1: Real-Time Map & Elevation Ingestion | Google Maps Directions & Elevation API integration | Completed | 100% |
| Core Task 2: Driving Cycle Synthesis for Simulink | Time-speed synthesizer & timeseries export | Completed | 100% |
| Core Task 3: Virtual Vehicle Composer BEV Modeling | Base workspace mapping & 10 Indian BEVs database | Completed | 100% |
| Core Task 4: Powertrain Simulation & Energy Accounting | Longitudinal forces, 2D motor map, battery ECM | Completed | 100% |
| Core Task 5: Visualization & Mapping Toolbox | Satellite/roadmap tiles, elevation & SOC plotting | Completed | 100% |
| Advanced Task 1: Driving Mode Optimization | Eco, Normal, Performance mode trade-off engine | Completed | 100% |
| Advanced Task 2: Highway Fast Charging Planner | Reserve violation detection, CC-CV taper model | Completed | 100% |

## 2. Implementation of Advanced Project Extensions

### 2.1 Advanced Extension 1: User-Informed Driving Modes (`optimizeDrivingMode.m`)

Three distinct driving profiles were calibrated to simulate behavioral impacts on powertrain efficiency:

1. ECO Mode:

- Acceleration limit: $a_{\text{max}} = 1.2\,\text{m/s}^2$

- Cruising speed cap: $v_{\text{cap}} = 85\,\text{km/h}$

- Regenerative braking prioritization: $100\%$ regen capture before mechanical friction brake engagement.

- Auxiliary HVAC load: Eco compressor setting ($0.8\,\text{kW}$).

2. NORMAL Mode:

- Acceleration limit: $a_{\text{max}} = 1.8\,\text{m/s}^2$

- Cruising speed cap: $v_{\text{cap}} = 100\,\text{km/h}$

- Standard regen blending: $80\%$ recovery efficiency.

- Auxiliary HVAC load: Standard comfort setting ($1.5\,\text{kW}$).

3. PERFORMANCE Mode:

- Acceleration limit: $a_{\text{max}} = 2.8\,\text{m/s}^2$

- Cruising speed cap: $v_{\text{cap}} = 120\,\text{km/h}$ (expressway legal ceiling)

- Dynamic sport regen: $60\%$ capture ratio with high wheel torque response.

- Auxiliary HVAC load: High cooling capacity ($2.2\,\text{kW}$).

#### Mode Comparison Results (Mumbai-Pune Corridor, Tata Nexon EV LR, 148 km):

| Driving Mode | Travel Time | Net Energy | Avg. Efficiency | Final SOC | Trip Cost | Energy Savings vs Performance |
| --- | --- | --- | --- | --- | --- | --- |
| ECO | 2h 14m | 21.4\,kWh | 14.4\,kWh/100km | 47.4\% | ₹\,192.60 | +27.2\% saved |
| NORMAL | 2h 08m | 25.1\,kWh | 16.9\,kWh/100km | 39.2\% | ₹\,225.90 | +14.6\% saved |
| PERFORMANCE | 1h 54m | 29.4\,kWh | 19.8\,kWh/100km | 29.6\% | ₹\,264.60 | Baseline |

ℹ **Key Finding:** Driving in Eco mode saves $8.0\,\text{kWh}$ of battery energy (equivalent to $45\,\text{km}$ of additional range) while only increasing travel time by 20 minutes across the 148 km trip.

### 2.2 Advanced Extension 2: Highway Fast Charging Optimization (`optimizeCharging.m`)

When a planned route exceeds the vehicle's safe single-charge radius, the platform automatically plans charging stops:

1. Reserve Violation Detection:

Monitors instantaneous battery State of Charge. If $\text{SOC}(t) < 15\%$, a violation flag is triggered.

2. Candidate Station Spatial Query:

Scans the corridor for verified DC Fast Charging stations (Tata Power EZ Charge, Jio-bp pulse, Zeon Electric, etc.).

3. Non-Linear CC-CV Taper Model:

Models constant-current (CC) high-speed charging up to 80% SOC followed by constant-voltage (CV) thermal taper, preventing unrealistic linear charging estimations:

$$t_{\text{charge}} = \frac{E_{\text{needed}}}{P_{\text{charger}} \cdot \eta_{\text{charge}} \cdot \kappa_{\text{taper}}}$$

Where $\kappa_{\text{taper}} = 0.85$ accounts for the slowing charge rate above 70% SOC.

4. Session Cost Calculation:

Computes electricity billing in INR based on commercial EV fast-charging tariffs ($₹\,18.50/\text{kWh}$).

## 3. Technical Challenges Overcome

| Challenge | Impact | Engineering Solution Implemented |
| --- | --- | --- |
| Noisy Raw Elevation Data | Step discontinuities from raw GPS/Google elevation caused unrealistic high-frequency grade spikes. | Implemented a spatial moving-average smoothing filter (sgolay / smoothdata) to produce realistic, smooth road slope angles (\theta). |
| API Rate Limits / Quotas | Network failures or missing API keys could crash simulations during offline grading. | Developed automated fallback to 6 high-resolution pre-compiled .mat Indian highway corridors. |
| Simulink Integration Friction | Manually importing custom cycles into Simulink was error-prone for evaluators. | Created export_route_to_simulink.m which automatically writes variables to base workspace and saves route_drive_cycle.mat. |

## 4. Work Remaining for Final Phase

1. Master Test Suite Integration: Assemble all unit and integration tests into a single master verification harness (runAllTests.m).

2. Comprehensive Final Documentation: Compile physical derivations into ModelAssumptions.md and SystemArchitecture.md.

3. Public GitHub Repository Deployment: Push codebase, configure clean README, and post official submission on MathWorks Challenge Hub Discussion #128.

4. Final Presentation & Slide Deck: Prepare slide materials for final project viva (LAB 9).